EGC221: Digital Logic Lab

Experiment #3

Combinational Logic Circuits

| Student’s Name: | Reg. no.: |
| --- | --- |
| Student’s Name: | Reg. no.: |
| Semester: Fall 2026 | Date: 29 July 2026 |

Assessment:

| Assessment Point | Weight | Grade |
| --- | --- | --- |
| Methodology and correctness of results |  |  |
| Discussion of results |  |  |
| Participation |  |  |
| Assessment Points’ Grade: |  |  |

Comments:

Experiment #3:

Basic Logic Circuits

Objectives:

The objectives of this experiment are to:

- Verify a logic function through circuit simulation, then by building the logic circuit,

- Analyze a logic circuit using a truth table, and

- Investigate the concept that two or more circuits can result in exactly the same output.

Procedure:

Use Logisim and Digital I/O Module, DC Power Supply, Breadboard, Wires, and 74xx Logic ICs to solve the following exercises.

Exercise 1:

- Derive the logic expression for the circuit illustrated in Figure 1.

Figure 1. Basic logic circuit with three inputs A, B, and C.

| Logic Expression: | X = |
| --- | --- |

- Analyze the circuit in Figure 1 and complete Table 1 below.

Table 1. Basic logic circuit with three inputs A, B, and C.

INPUTS				    INTERMEDIATES		OUTPUT

| A | B | C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  |  |  |  |  |
| 0 | 0 | 1 |  |  |  |  |  |
| 0 | 1 | 0 |  |  |  |  |  |
| 0 | 1 | 1 |  |  |  |  |  |
| 1 | 0 | 0 |  |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |
| 1 | 1 | 1 |  |  |  |  |  |

- Use the breadboard in Figure 2 and mark (using a color pen or pencil or import to Paint) all the necessary connections to build the circuit shown in Figure 1. Connect Input A on switch S7, Input B on switch S6, Input C on switch S5, and connect output X on LED 7. [Note: Use a ruler to draw the lines connecting the components and clearly indicate connection points.]

Figure 2. Breadboard diagram for logic function X.

- Use Logisim to simulate the ICs and connections shown in Figure 2.  Complete Figure 3 and Table 2.

[Insert Logisim circuit here]

Figure 3. 7404, 7408 and 7432 ICs circuit simulation

Table 2. Basic logic circuit – Logisim results.

| S7 | S6 | S5 | L7 |
| --- | --- | --- | --- |
| 0 | 0 | 0 |  |
| 0 | 0 | 1 |  |
| 0 | 1 | 0 |  |
| 0 | 1 | 1 |  |
| 1 | 0 | 0 |  |
| 1 | 0 | 1 |  |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |

- Verify the operation of your circuit using a Digital I/O Module, DC Power Supply, Breadboard, 7404, 7408 and 7432 ICs, and Wires. Complete Figure 4 and Table 3.

[Insert Photo of circuit here]

Figure 4. Combinational circuit photo

Table 3. Basic logic circuit – Build results.

| S7 | S6 | S5 | L7 |
| --- | --- | --- | --- |
| 0 | 0 | 0 |  |
| 0 | 0 | 1 |  |
| 0 | 1 | 0 |  |
| 0 | 1 | 1 |  |
| 1 | 0 | 0 |  |
| 1 | 0 | 1 |  |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |

- Compare the results from Tables 1, 2, and 3:

Exercise 2:

- Use Logisim to simulate the logic expression: X = AC + B.  Complete Figure 5 and Table 4.

[Insert Logisim circuit here]

Figure 5. Basic circuit simulation of the logic expression: X =  AC + B.

Table 4. Basic Logic Circuit – Logisim results.

| S7 | S6 | S5 | L7 |
| --- | --- | --- | --- |
| 0 | 0 | 0 |  |
| 0 | 0 | 1 |  |
| 0 | 1 | 0 |  |
| 0 | 1 | 1 |  |
| 1 | 0 | 0 |  |
| 1 | 0 | 1 |  |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |

- Use the breadboard in Figure 6 and mark (using a color pen or pencil or import to Paint) all the necessary connections to build the circuit illustrated in Figure 4. Connect Input A on switch S7, Input B on switch S6, Input C on switch S5, and connect output X on LED 7. [Note: Use a ruler to draw the lines connecting the components and clearly indicate connection points.]

Figure 6. Basic circuit wiring of the logic expression X =  AC + B.

- Verify the operation of your circuit using a Digital I/O Module, DC Power Supply, Breadboard, 7408 and 7432 ICs, and Wires. Complete Figure 7 and Table 5.

[Insert Photo of circuit here]

Figure 7. Combinational Circuit.

Table 5. Basic Logic Circuit – Build results.

| S7 | S6 | S5 | Output |
| --- | --- | --- | --- |
| 0 | 0 | 0 |  |
| 0 | 0 | 1 |  |
| 0 | 1 | 0 |  |
| 0 | 1 | 1 |  |
| 1 | 0 | 0 |  |
| 1 | 0 | 1 |  |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |

- Compare the results from Tables 4 and 5:

- Conclusions (discussion of results):