EGC221: Digital Logic Lab

Experiment #5

Arithmetic Circuits Using Altera Quartus Prime

| Student’s Name: | Reg. no.: |
| --- | --- |
| Student’s Name: | Reg. no.: |
| Semester: Spring 2026 | Date: 31 July 2026 |

Assessment:

| Assessment Point | Weight | Grade |
| --- | --- | --- |
| Methodology and correctness of results |  |  |
| Discussion of results |  |  |
| Participation |  |  |
| Assessment Points’ Grade: |  |  |

Comments:

Experiment #5:

Full Adder Circuit Implementation

Objectives:

The objectives of this experiment are to:

- Get familiar with Altera Quartus Pime

- implement a full-adder using logic gates,

- implement a 4-bit ripple carry adder (using logic gates), and

- Modify the 4-bit adder such that it is capable of 4-bit addition/subtraction.

Procedure:

1. Become familiar with Altera Quartus Prime toolset by going through the Tutorial on Quartus Prime Schematic Capture.

2. Use Altera’s Quartus Prime Schematic to solve Exercise 1.

Exercise 1:

- Given Table 1 below, complete the truth table for a full adder, and derive the logic expression for Sum and Cout.

Table 1. 1-bit Full Adder using basic gates truth table

| A | B | Cin | Sum | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  |  |
| 0 | 0 | 1 |  |  |
| 0 | 1 | 0 |  |  |
| 0 | 1 | 1 |  |  |
| 1 | 0 | 0 |  |  |
| 1 | 0 | 1 |  |  |
| 1 | 1 | 0 |  |  |
| 1 | 1 | 1 |  |  |

| Logic Expression: | Sum: |
| --- | --- |
| Logic Expression: | Cout: |

- Use Quartus Prime Schematic to provide the full-adder circuit diagram:

[Insert circuit diagram here]

Figure 1. Quartus Prime circuit diagram of full-adder

- Use Quartus Prime Schematic to provide functional and timing verifications:

[Insert functional verification timing diagram here]

[Insert timing verification timing diagram here]

Figure 2. Quartus Prime simulations of full adder

(functional is shown on top, and timing below)

Next, create a block diagram of the design in Figure 1 per Quartus Prime Schematic tutorial and continue with Exercise 2.

Exercise 2:

Using the circuit design from exercise 1 (Full Adder) and a hierarchical approach (as shown in Figure 3 below), create a 4-bit Binary Ripple Adder.

Figure 3. Hierarchical block diagram of 4-bit adder

- Use Quartus Prime Schematic to provide the 4-bit adder diagram:

[Insert circuit diagram here]

Figure 4. Quartus Prime circuit diagram of 4-bit adder

- Use Quartus Prime Schematic to provide functional and timing verifications:

[Insert functional verification timing diagram here]

[Insert timing verification timing diagram here]

Figure 5. Quartus Prime simulations of 4-bit adder

(functional is shown on top, and timing below)

Exercise 3:

Modify the 4-bit adder from part (d), to implement a 4-bit adder/subtractor.

- Use Quartus Prime Schematic to provide the 4-bit adder/subtractor circuit diagram:

[Insert circuit diagram here]

Figure 6. Quartus Prime circuit diagram of 4-bit adder/subtractor

- Use Quartus Prime Schematic to provide functional and timing verifications:

[Insert functional verification timing diagram here]

[Insert timing verification timing diagram here]

Figure 7. Quartus Prime simulations of 4-bit adder/subtractor circuit

(functional is shown on top, and timing below)

Finally, verify the circuit by downloading on Altera DE10 board and have it signed by your instructor or TA.

Conclusions: