EGC221: Digital Logic Lab

Experiment #6

Hierarchical Logic Circuits Using Intel Quartus Prime

| Student’s Name: | Reg. no.: |
| --- | --- |
| Student’s Name: | Reg. no.: |
| Semester: Spring 2026 | Date: 30 July 2026 |

Assessment:

| Assessment Point | Weight | Grade |
| --- | --- | --- |
| Methodology and correctness of results |  |  |
| Discussion of results |  |  |
| Participation |  |  |
| Assessment Points’ Grade: |  |  |

Comments:

Experiment #6:

Hierarchical Logic Circuit Implementation

Objectives:

The objectives of this experiment are to:

- implement (and simulate) a 2 x 1 Multiplexer using logic gates,

- create a (block) symbol of the 2 x 1 Multiplexer, and

- use block symbols (a hierarchical approach) of the 2 x 1 to implement (and simulate) an 8 x 1 Multiplexer.

Procedure:

Use Intel’s Quartus Prime Schematic to solve the following exercises.

Exercise 1:

In this section, a new Quartus® Prime project will be created to design an 8 x 1 Multiplexer circuit, the top-level module. Follow the steps shown in part I of this tutorial to create a new project using New Project Wizard in Quartus® Prime.

Choose “MUX_8x1” as the name for both the project and the top-level entity. It is important to notice that the name of the top-level entity must match the entity name in the design file.

We will start by designing a 2 x 1 Multiplexer using schematic editor. A Multiplexer uses a control input to select a single data input (from several choices) to be routed to the output of the circuit. If the control SEL is low, then output Y = I0; else,Y = I1.

Table 1. Truth table for a 2 x 1 Mux.

| SEL | Y |
| --- | --- |
| 0 | I0 |
| 1 | I1 |

Figure 1 illustrates the block diagram of a 2 x 1 Multiplexer.  Design its internal circuit using basic AND and OR gates. Be sure to save as “MUX_2x1” and Set as Top-Level Entity under the Projects pull down menu.

I0

Y

I1  
 SEL

Figure 1. Block diagram of 2 x 1 MUX

- Use Quartus Prime Schematic to provide the 2 x 1 Multiplexer circuit diagram

[Insert circuit diagram here]

Figure 2. Quartus Prime circuit diagram of 2 x 1 MUX

- Use Quartus Prime Schematic to provide functional verification.

[Insert functional verification timing diagram here]

Figure 3. Quartus Prime functional simulation of 2 x 1 MUX

Exercise 2:

Create a block symbol to represent the above file:

Click inside the mux_2x1.bdf to make sure this is the active window.

Click File -> Create/Update -> Create Symbol Files for Current File.

Exercise 3:

Create a new schematic diagram file:

Save this file as mux_8x1.bdf

An 8 X 1 Mux has the following truth table.

Table 2. Truth table for an 8 x 1 Mux.

| S2 S1 S0 | Z |
| --- | --- |
| 0 0 0 | I0 |
| 0 0 1 | I1 |
| 0 1 0 | I2 |
| 0 1 1 | I3 |
| 1 0 0 | I4 |
| 1 0 1 | I5 |
| 1 1 0 | I6 |
| 1 1 1 | I7 |

Using the Symbol Tool, and under Project you will find your new symbol. Using 2 X 1 Mux’s, create the 8 x 1 MUX circuit in Quartus Prime.

Click Project -> Set as Top Level Entity

[Insert circuit diagram here]

Figure 4. Quartus Prime circuit diagram of 8 x 1 MUX

Use Quartus Prime Schematic to provide functional and timing verifications.

[Insert functional verification timing diagram here]

[Insert timing verification timing diagram here]

Figure 5. Quartus Prime simulations of 8 x 1 MUX symbol.

(Functional is shown on top, and timing below)

Download and verify a working 8 x 1 MUX

Conclusions: